Product Guide Specification

Specifier Notes:  This product guide specification is written according to the Construction Specifications Institute (CSI) 3-Part Format, based on MasterFormat 2016 and The Project Resource Manual—CSI Manual of Practice. The Manufacturer is responsible for technical accuracy.

The section must be carefully reviewed and edited by the Architect or Engineer to meet the requirements of the project and local building code.  Words and sentences within brackets [ ] are choices to include or exclude a particular item or statement.  Coordinate this section with other specification sections and the Drawings.  Delete all “Specifier Notes” after editing this section.

Electronic Safety and Security

Common Work Results for Electronic Safety and Security

Network Video Recorders

Intelligent Video Surveillance Server

- – GENERAL

  - SUMMARY

    - Section Includes

      - Storage Area Network Electronic Safety and Security

      - Cloud Based Storage for Electronic Safety and Security

      - Storage Management Software for Electronic Safety and Security

      - Communications Equipment for Electronic Safety and Security

    - Related Sections

      - [Security Detection, Alarm and Monitoring – Security Monitoring and Control – Security Monitoring and Control Software].

**********Specifier’s note: Include those standards referenced elsewhere in this SECTION.

  - REFERENCES

| STANDARD | CE: CE-LVD: EN 60950-1/IEC 60950-1 CE-EMC: EN 61000-3-2, EN 61000-3-3, EN 55032, EN 50130, EN 55024 FCC: Part 15 Subpart B UL 60950-1 |
| --- | --- |

  - SYSTEM DESCRIPTION

    - Section Includes

      - Intelligent Video Surveillance Server

  - SUBMITTALS

    - Product Data:

      - Manufacturer’s data, user and installation manuals for all equipment and software programs including computer equipment and other equipment required for complete video management system.

    - Dimensional Drawings; include

      - Overall device dimensions.

      - Dimensions specific for installation.

    - Closeout Submittals

      - User manual.

      - Parts list.

      - Maintenance requirements.

  - QUALITY ASSURANCE

    - Manufacturer:

      - Minimum of [10] years of experience in manufacture and design Video Surveillance Devices.

    - Video Surveillance System:

      - List certifying bodies (FCC,CE, etc.)

      - Provide evidence of compliance upon request.

    - Installer:

      - Minimum of [5] years of experience installing Video Surveillance System.

  - DELIVERY, STORAGE AND HANDLING

    - Comply with requirements.

    - Deliver materials in manufacture’s original, unopened, undamaged containers; and unharmed original identification labels.

    - Protect store materials from environmental and temperature conditions following manufacturer’s instructions.

    - Handle and operate products and systems according to manufacturer’s instructions.

  - WARRANTY

    - Provide manufacturer’s warranty covering [3] years for replacement and repair of defective equipment. Warranty varies country to country.

  - MAINTENANCE

    - Make ordering of new equipment for expansions, replacements, and spare parts available to dealers and end users.

    - Provide factory direct technical support via phone and e-mail.

- – PRODUCTS

  - MANUFACTURERS

    - Acceptable Manufacturer:

ZHEJIANG DAHUA VISION TECHNOLOGY CO., LTD.

Address: No.1399, Binxing Road, Binjiang District, Hangzhou, P.R. China

Postcode: 310053

Tel: +86-571-87688888 28933188

Email: dhoverseas@dhvisiontech.com

Website: www.dahuasecurity.com

Substitutions:

      - [All proposed substitutions must be approved by the Architect or Engineer professional.]

      - [Proposed substitutions must provide a line-by-line compliance documentation.]

  - Intelligent Video Surveillance Server | DHI-IVSS7016-4I

| System | Main Processor | Industry embedded micro-controller |
| --- | --- | --- |
| System | Operating System | Embedded Linux OS |
| System | Operating Interface | Web (PCAPP), local GUI |
| Face Detection | Face Detection | 1. AI by camera: Supports. 2. AI by device: Supports. Supports detection zone settings, the event can trigger record, snapshot storage (full image), local alarm output, IPC external alarm output, access control, audio prompt, buzzer, preset, email. |
| Face Recognition | Face Database Capacity | 1. Up to 50 human face databases with maximally 300 thousand face images in total, which means that one face database can contain up to 300 thousand face images. 2. Up to 5 passerby databases with maximally 300 thousand face images in total, which means that one passerby database can contain up to 300 thousand face images. The human face database and the passerby database share the total capacity and number of face dimages. |
| Face Recognition | Face Recognition | 1. Recognition mode: front FR, front FD+Back FR, front FR+back FR, back FD+back FR. 2. AI by Camera: Supports. 3. AI by Device: Function configurations: Supports arming face database and passerby database. Supports enabling general/stranger mode. Supports deploying several human face databases and setting similarity separately. Once the similarity reaches or does not reach the (stranger) threshold, the client software can generate the alarm prompt and trigger record, snapshot storage (full image), local alarm output, IPC external alarm output, access & control, audio prompt, buzzer, log, preset, email. |
| Vehicle License Plate Comparison | License Plate Database Capacity | 1. Max. 50 plate databases. One database contains up to 3 hundred thousand plate numbers. 2. Supports black list/white list. |
| Vehicle License Plate Comparison | Vehicle License Plate Comparison | Enables plate recognition after adding the ITC camera. Function settings: Deploys several plate databases, once the device detects the plate is matching the information on the plate database (black list/white list), the client can output alarm prompt and then trigger record, snapshot (full image), local alarm output, IPC external alarm output, access and control, audio prompt, buzzer, log, preset, email. |
| IVS Analytics | IVS | 1. AI by camera: Tripwire, intrusion, abandoned object, missing object, fast moving, crowd gathering, loitering, parking, and fence-crossing. 2. AI by device: The general model supports tripwire and intrusion. The advanced model supports tripwire, intrusion, crowd gathering, parking detection, loitering. 10 rules at most. Configuration: When the relevant alarms are triggered, the system responses with linked actions such as PCAPP client message notification, video recording, snapshot storage (background picture), alarm output from IPC or the device, access control, audible warning, buzzer, logs, PTZ preset, and email. |
| Audio and Video | Decoding Capability | 2-channel 24 MP; 3-channel 16 MP; 6-channel 12 MP@20 fps; 7-channel 12 MP@15 fps; 6-channel 8 MP@30 fps; 12-channel 8 MP@15 fps; 7-channel 6 MP@30 fps; 9-channel 5 MP@30 fps; 11-channel 5 MP@25 fps; 12-channel 4 MP@30 fps; 18-channel 4 MP@20 fps; 19-channel 3 MP@25 fps; 12-channel 1080p@60 fps; 24-channel 1080p@30 fps; 36-channel 720p@30 fps |
| Network | Network Mode | Multiple-address mode; Load balance; Fault-tolerance; Link aggregation |
| Audio and Video | Access Channel | 256-channel |
| Audio and Video | Network Bandwidth | Incoming bandwidth: 512Mbps, recording bandwidth: 384Mbps, outgoing bandwidth: 128Mbps |
| Audio and Video | Video Output | 1-channel VGA output, 3-channel HDMI output, VGA 1/HDMI 1 outputs the same video source. Supports 4K output. |
| Audio and Video | Multi-screen Display | Customized display. Max. 36-channel local live view. Max.16-channel PCAPP live view. |
| Storage | RAID | RAID0; RAID1; RAID5; RAID6; RAID10; RAID50; RAID60 |
| Audio and Video | Third-party Camera Access | Onvif, RTSP, Sony, Panasonic, Axis, Arecont, Pelco,Canon, Samsung |
| Network | Mobile Phone Access | DMSS |
| Network | Interoperability | ONVIF (profile S), CGI, SDK |
| Network | Browser | Chrome, PCAPP, IE9 or higher, Firefox |
| Recording Playback | Multi-channel Playback | Max. 16-channel playback |
| Recording Playback | Record Mode | Search video detection/manual record/IO alarm/intelligent event/all record file |
| Recording Playback | Storage Method | Support internal HDD and SAS disk array enclosure. |
| Recording Playback | Backup Method | HDD, peripheral USB storage device |
| Recording Playback | Playback Function | 1. Play/pause/stop/fast forward/fast backward/reverse play/frame by frame play. 2. Full-screen, backup (clip/file), snapshot, digital zoom, audio on/off. |
| Storage | Disk Group | 16 |
| Storage | Record Management | Supports record control (continuous, event-based, scheduled, scheduled & event-based). |
| Alarm | General Alarm | Motion detect, tampering, local alarm, IPC external alarm, fire alarm, temperature alarm, temperature difference alarm, hot spot alarm, and cold spot alarm |
| Alarm | Anomaly Alarm | IPC offline alarm, storage error, HDD full, SSD exception, IP conflict, MAC conflict, login lock, AI module temperature alarm, AI module offline, fan malfunction, no HDD, network security exception, and power exception. |
| Alarm | Intelligent Alarm | Human face detection, human face recognition, video metadata (person/vehicle/Non-motor vehicle), metadata attributes alarm by device，IVS,plate recognition, people number alarm, stay number alarm, queue alarm. |
| Alarm | Alarm Linkage | Record, snapshot (full image), local external alarm output, uploading alarms, IPC external alarm output, access & control, audio prompt, buzzer, log, preset, email，smart tracking. |
| Port | eSATA | 1 port |
| Port | SAS | 2 SAS3.0 ports |
| Port | RS-232 | 1 port, for debug or COM data transmission |
| Port | RS-485 | 1 port, control peripheral PTZ and so on. Supports various protocols. |
| Port | USB | 4 ports. 2 USB2.0 port at the front panel and 2 USB3.0 port a the rear panel. |
| Port | HDMI | 3 ports. Support 4K output. |
| Port | VGA | 1 port |
| Port | Network Port | 4 RJ-45 10/100/1000Mbps self-adaptive Ethernet ports |
| Intelligent Task Analytics | Intelligent Task Analytics | 1. Video source: Local video records,import DAV videos from external devices. 2. Execution time: Set the start and end time of the task for each day. 3. Result search: Support searching by attributes, by image and by task. 4. Performance: 1-channel analytics at one time. |
| Thermal | Thermal | 1. Alarms by Camera: Fire alarm, temperature alarm, temperature difference alarm, hot spot alarm, cold spot alarm. 2. Search: Supports searching for thermal alarms. 3. Linkage actions: Video recording, snapshot storage, alarm output from IPC or the device, audible warning, buzzer, logs, PTZ preset, and email. |
| Fisheye Dewarp | Fisheye Dewarp | 1. Supports fisheye dewarp by the device for up to 3 fisheye cameras. 2. Fisheye mounting mode: Ceiling mount, wall mount, and ground mount. 3. Dewarp mode: 7 modes for ceiling mount: 360°1P+1, 2P,1+3,1+4,1P+6,1+8; 5 modes for wall mount:360°，1P, 1P+3，1P+4，1P+8; 7 modes for ground mount: 360°1P+1, 2P,1+3,1+4,1P+6,1+8. For a 180° fisheye camera, the mounting mode is wall mount by default and cannot be changed. It supports 5 modes: 180°，1P, 1P+3，1P+4，1P+8. |
| People Counting | People Counting | 1. Alarms: People counting, in area No., queque alarm. 2. Reports: People counting, in area No., and queque report by day or month. Supports searching for reports by area, type, and stay time. Reports can be exported. |
| Port | Audio Input | 1 channel |
| Port | Audio Output | 2 channels |
| Bullet-PTZ Smart Track | Bullet-PTZ Smart Track | Working with cameras to support manual tracking, automatic tracking, positioning by click, positioning by frame selection. |
| Port | Alarm Input | 16 channels |
| Cluster | Cluster | Supports N+M cluster. |
| Port | Alarm Output | 8 channels |
| AI | AI by Camera | Supports AI by camera function: Human face detection, human face recognition, video metadata (person/vehicle/Non-motor vehicle), vehicle recognition, perimeter protection |
| Port | Disk Interface | 16 slots, SATA3.0/SAS. Max.16 TB/HDD |
| AI | AI by Recorder | Supports AI by IVSS function: Human face detection, human face recognition, entries frequency alarm, video metadata (person/vehicle/non-motor vehicle), metadata attributes alarm (AI by Recorder), IVS (Tripwire, Intrusion, Fast moving, Crowd Gathering, Loitering Detection, Parking), Plate recognition. |
| Port | Power | Single supply |
| General | Disk Bay | 16 |
| General | Form Factor | 3U |
| General | Power Supply | 100–127/200–240 VAC, 50/60 Hz |
| General | Power Consumption | 100 W (without HDD, idling) 292 W（all HDDs connected） |
| General | Net Weight | 16 kg |
| General | Gross Weight | 17 kg |
| General | Product Dimensions | Chassis: 444.8 mm × 522.4 mm × 133.2 mm (W × L × H). Max. size: 485 mm (with hanger) × 522.4 mm × 133.2 mm (with feet mat) (W × L × H) |
| General | Packaging Dimensions | With Packaging: 689 mm × 779 mm × 464 mm (W × L × H). |
| General | Operating Temperature | 0 °C to +45 °C (+32 °F to +113 °F) |
| General | Operating Humidity | 10%–90% (RH), non-condensing |
| General | Operating Altitude | 5,000 m |
| General | Installation | Desktop; rack |

- – EXECUTION

  - EXAMINATION

    - Examine areas to receive devices and notify adverse conditions affecting installation or subsequent operation.

    - Do not begin installation until unacceptable conditions are corrected.

  - PREPARATION

    - Protect devices from damage during construction.

  - INSTALLATION

    - Install devices in accordance with manufacturer’s instruction at locations indicated on the floor drawings plans.

    - Perform installation with qualified service personnel.

    - Install devices in accordance with the National Electrical Code or applicable local codes.

    - Ensure selected location is secure and offers protection from accidental damage.

    - Location must provide reasonable temperature and humidity conditions, free from sources of electrical and electromagnetic interference.

  - FIELD QUALITY CONTROL

    - Test snugness of mounting screws of all installed equipment.

    - Test proper operation of all video system devices.

    - Determine and report all problems to the manufacturer’s customer service department.

  - ADJUSTING

    - Make proper adjustment to video system devices for correct operation in accordance with manufacturer’s instructions.

    - Make any adjustment of camera settings to comply with specific customer’s need.

  - DEMOSTRATION

    - Demonstrate at final inspection that video management system and devices functions properly.

END OF SECTION